| ##### 24HOURS PROGRAMMABLE ##### ELECTRONIC TIMER SWITCH |  |
| --- | --- |
| - DIN-sized 35mm Rail Installation. - Latest technology COMS quartz Mico-controller - Digital electronic general purpose time switch with daily programs - Minimum interval:15minutes; easy-to-use. - Battery inside provides power if the out power is cut. - To-the-minute setting accuracy. - LCD displays real time to Hour/Minute and ON/AUTO/OFF. - This product owns the patent.. |  |

## ■TECHNICAL  DATA

| ●Voltage limit： ●Hysteresis： | AC180V ~ 250V (50/60Hz) ≤1s/d (at25℃) | ●Number of circuits： ●Load capacity： | 1c 16A 250VAC（ohmic load cos Ø=1） |
| --- | --- | --- | --- |
| ●On/off operation： ●Minimum interval： | 48on & 48off 15 minutes | ●Service life： | Mechanically: 107 Electrically: 105 |
| ●Weight： | Approx 200g | ●Ambient temperature | -10～55℃ |
| ●Display； | LCD | ●storage temperature: | -20~70℃ |
| ●Battery backup: | 150h | ●Power consumption： | 5VA |

|  | ■DIMENSIONS ■CONNECTIONS |  |  |  |
| --- | --- | --- | --- | --- |

■ OPERATING  INSTRUCTION:

1. If the timer switch is used for the first time or unused for long time, and it dose not displays,please input AC and let battery recharge .After then press reset key in the small hole which is located in the right-up corner of the panel.

2. Programming  :

| Step | Key | Programming |
| --- | --- | --- |
| 1 | Press P key | Begin Setting |
| 2* | Press + or - key | Select on-time |
| 3 | Press ok key | Set on-time and Press ok to confirm(If you want to add next 15minutes to on-time,you need to press this key again) |
| 4 | Repeat 2-3 | Setting 2-48 on/off-time |
| 5 | Press P key | End |

3. CLOCK CALIBRATE

(1)  Press “P” key  for 2 seconds  and the  hour bit should be glinting.

- Press “+” key or” - ” key to set the hour of the real time

- Press   ok key again and the  minute bit should be glinting.

- Press “+” key or” - ” key to set the minitue of the real time

4.AMEND THE ON/OFF TIME

| Step | Key | Programming |
| --- | --- | --- |
| 1 | Press P key | Begin Setting |
| 2* | Press + or - key | Select the on-time which will be cancled |
| 3 | Press ok key | Cancle the on-time,one time for 15minutes,and the key can press continuely. |
| 4 | Press P key | End |

5. If you want to use time switch  ON or OFF at random,Please Press “ ok ”key to display “ON” or “OFF”.

6.Setting daylight saving time

Press “P” key and  “+ ” key  together, the real time can be added 1 hour.

■NOTE:

1.PLEASE USE IT IN THE CONDITION OF RATING VOLTAGE, REGULAR TEMPERATURE AND HUMIDITY, AND STOR IT IN REGULAR TEMPERATURE AND HUMIDITY.

2.PLESE DO NOT USE IT IN THE ENVIROMENT OF MUCH DUST,ERODED GAS, SOLARIZATED  BY SUNLIGHTE DIRECTELY,OR DRENCHED BY RAIN.